Listing 1

Firmware structure for Experiment 1: temperature-gradient measurement and monitoring

/****************************************************
Listing 1.
Core firmware structure for Experiment 1:
temperature-gradient measurement and monitoring
Controller: NodeMCU ESP8266
****************************************************/

#include <ESP8266WiFi.h>
#include <BlynkSimpleEsp8266.h>// Library V 1.0.1
#include <MAX6675.h>//Library Robb Tillaat v 0.1.1

/* -------- Pin mapping -------- */
// MAX6675 #1 (hot side)
const int clockPin = D5;
const int dataPinHot = D6;
const int csPinHot = D1;

// MAX6675 #2 (cold side)
const int dataPinCold = D7;
const int csPinCold = D2;

/* -------- Sensor initialization -------- */
MAX6675 thermocoupleHot;
MAX6675 thermocoupleCold;

/* -------- Temperature acquisition and processing -------- */
void readTemperature()
{
float T1 = thermocoupleHot.read();
float T2 = thermocoupleCold.read();
float deltaT = T1 - T2;

Blynk.virtualWrite(V0, T1);
Blynk.virtualWrite(V1, T2);
Blynk.virtualWrite(V2, deltaT);
}

void setup()
{
thermocoupleHot.begin(clockPin, csPinHot, dataPinHot);
thermocoupleCold.begin(clockPin, csPinCold, dataPinCold);
}

Listing 2

Firmware structure for Experiment 2: electrical characterization and extended logging

/****************************************************
Listing 2.
Core firmware structure for Experiment 2:
electrical characterization and extended logging
Controller: WeMos D1 mini ESP8266
****************************************************/

#include <ESP8266WiFi.h>
#include <BlynkSimpleEsp8266.h> // Library V 1.0.1
#include <MAX6675.h> //Library Robb Tillaat v 0.1.1
#include <ModbusMaster.h>//Author:: Doc Walker <4-20ma@wvfans.net>  V 2.0.1
#include <SoftwareSerial.h>

/* -------- Pin mapping -------- */
// MAX6675 #1 (hot side)
const uint8_t thermoSCK = D7;
const uint8_t thermoSO = D6;
const uint8_t thermoCS1 = D2;

// MAX6675 #2 (cold side)
const uint8_t thermoCS2 = D1;

// MAX485 / RS485 interface for PZEM-017
const uint8_t MAX485_RO = D5;
const uint8_t MAX485_DI = D3;
const uint8_t MAX485_RE = D0;
const uint8_t MAX485_DE = D4;

/* -------- Sensor initialization -------- */
MAX6675 thermocoupleHot;
MAX6675 thermocoupleCold;

SoftwareSerial pzemSerial(MAX485_RO, MAX485_DI);
ModbusMaster node;

/* -------- RS485 control -------- */
void preTransmission()
{
digitalWrite(MAX485_RE, HIGH);
digitalWrite(MAX485_DE, HIGH);
}

void postTransmission()
{
digitalWrite(MAX485_RE, LOW);
digitalWrite(MAX485_DE, LOW);
}

/* -------- Measurement acquisition and processing -------- */
void readSystem()
{
float T_hot = thermocoupleHot.read();
float T_cold = thermocoupleCold.read();
float deltaT = T_hot - T_cold;

uint8_t result = node.readInputRegisters(0x0000, 6);
if (result == node.ku8MBSuccess)
{
float voltage = node.getResponseBuffer(0) / 100.0;
float current = node.getResponseBuffer(1) / 100.0;
float power = node.getResponseBuffer(3) / 10.0;
float energy = node.getResponseBuffer(5) / 1000.0;

Blynk.virtualWrite(V0, T_hot);
Blynk.virtualWrite(V1, T_cold);
Blynk.virtualWrite(V2, deltaT);
Blynk.virtualWrite(V3, voltage);
Blynk.virtualWrite(V4, current);
Blynk.virtualWrite(V5, power);
Blynk.virtualWrite(V6, energy);
}
}

void setup()
{
pinMode(MAX485_RE, OUTPUT);
pinMode(MAX485_DE, OUTPUT);

thermocoupleHot.begin(thermoSCK, thermoCS1, thermoSO);
thermocoupleCold.begin(thermoSCK, thermoCS2, thermoSO);

pzemSerial.begin(9600);
node.begin(1, pzemSerial);
node.preTransmission(preTransmission);
node.postTransmission(postTransmission);
}